# INSTRUCTIONS TO E-MAIL FROM HOMEVISION OR HOMEVISION-PRO

Introduction

These instructions show a simple way to have HomeVision or HomeVision-Pro (both referred to hereafter as “HomeVision”) send an e-mail.  It requires that the HomeVision controller be connected to a PC running the HomeVision software.  The PC must be connected to the Internet.  It does not require an e-mail program or any other program to be running.  This method was by developed by Gary Kuznitz, a HomeVision user.

## Steps

- Place the EmailSend.bat and EmailNotice.vbs files into the Homevision folder.

- Modify the EmailNotice.vbs file.  Search for ### to locate the three places that require changing with a text editor.  They are described below:

- Change "to@example.com" to the email address you want to send the e-mail to.

- Change "mail.mymailserver.net" to any email server.  It can be your ISP.

- Change "MyCompany <from@example.com>" to the email address you want to show the email was from.

- To send an email from HomeVision, create a macro that transmits an ASCII string in the following format:

run program "EmailSend.bat /Interesting Subject /Body of the email /me@example.com /C:\Program Files\HomeVision\051020.log"

Note:  The above is all on one line.

This ASCII string will cause the HomeVision software to run the EmailSend.bat file.  The text following “EmailSend.bat” are four parameters that will be passed to EmailSend.bat.  Each parameter is separated by a slash "/". The four parameters are:

- Subject

- Body Text

- BCC (Can have one email address or two with the format me@example.com;work@example.com)

- Attachment (Must include full path)

You must have a minimum of one parameter (Subject); the rest are optional.

Note that you could transmit the entire ASCII string with a single “Transmit ASCII string ####” command.  However, you can alternatively use a series of commands that transmit different parts.  This allows you to include things like variable values (such as the HVAC setpoint) or the state of an X-10 device.  See the “Tips for using the serial interface” note on the HomeVision web site’s Application Notes page (http://www.csi3.com/hv_app.htm) for more details of transmitting serial data.

You can test a one-line command using the “Serial Command Test” screen under the “Other” menu.  Just enter the command (i.e, run program "EmailSend.bat /Subject /Body /me@example.com /C:\Program Files\HomeVision\051020.log") into the “Serial command from controller to test” field and click “Try It”.

- Set up your schedule to run the macro whenever you want to send an e-mail.

- Download the schedule into the HomeVision controller.

- Test it.

Note that if you run into troubles, the EmailSend.bat file can help you debug it.  It contains some lines to help in debugging, but that are currently commented out (commented lines start with “::”).  You can temporarily remove them to see exactly what is being passed to the BAT file to verify it is correct.