This file shows the format of the serial interface commands.  Refer to the file "Serial" for more information.

TABLE 1    CONTROLLER COMMANDS FORMAT (VERSION 2.62)

| COMMAND |  |  |  | RESPONSE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION | CHARACTER / ASCII VALUE | LEN | DATA | ID # | DATA | NOTES |
| X-10 Command - 4 bytes | X / 58h | 4 | 1/2: module ID # 3/4: command # | 28 | Various | See appendix for command details |
| X-10 Command - 6 bytes | P / 50h | 6 | 1/2: module ID # 3/4: command # 5/6: Ò##Ó data value | 20 | No Data | See appendix for command details |
| X-10 Command - 8 bytes | g / 67h | 8 | 1/2: module ID # 3/4: command # 5/6: data byte 1 7/8: data byte 2 | 37 | No Data | See appendix for command details |
| Input Port Command | : / 3Ah | 4 | 1/2: port ID # 3/4: command # | 0A | No Data | See appendix for command details |
| Output Port Command - 4 bytes | 3 / 33h | 4 | 1/2: port ID # 3/4: command # | 03 | No Data | See appendix for command details |
| Output Port Command - 6 bytes | M / 4Dh | 6 | 1/2: output port ID # 3/4: command # 5/6: Ò##Ó data value | 1D | No Data | See appendix for command details |
| Output Port Command - 8 bytes | Z / 5Ah | 8 | 1/2: output port ID # 3/4: command # 5/6: Ò##Ó value MSB 7/8: Ò##Ó value LSB | 2A | No Data | See appendix for command details |
| IR Command | ; / 3Bh | 4 | 1/2: IR signal ID # 3/4: command # | 0B | No Data | See appendix for command details |
| IR Command to Transmit Multiple Times | < / 3Ch | 4 | 1/2: IR signal ID # 3/4: # of times to xmit | 0C | No Data | See appendix for command details |
| Transmit Standard IR Signal | T / 54h | 4 | 1/2: device code 3/4: key code | 24 | No Data |  |
| Controller Command | G / 47h | 2 | 1/2: command # | 17 | Various depending on command | See appendix for command details |
| Video Command - 4 bytes | L / 4Ch | 4 | 1/2: command # 3/4: Ò##Ó data value | 1C | No Data | See appendix for command details |
| Video Command - 15 bytes | d / 64h | 15 | 1 : # of characters to write to screen 2-15: characters to write | 34 | No Data | See appendix for command details |
| System Variable Command | ^ / 5Eh | 6 | 1/2: command # 3/4: variable data type 5/6: ## value | 2E | See appendix for response details | See appendix for command details |
| Simulate Infrared Remote Keypress | J / 4Ah | 1 | 1: IR remote button code | 1A | No Data | See appendix for command details |