TABLE 2 - CONTROLLER MESSAGE FORMAT (VERSION 2.62)

| ID # | DESCRIPTION | NOTES | EXAMPLES |
| --- | --- | --- | --- |
| 80 | Power restored | Transmitted every time power is applied to unit. | 80 Power Restored. {CR}{LF}{SOH} |
| 81 | Self-test passed | The self-test is run after power is applied. This message is transmitted if it passes. | 81 Self-Test Passed!{CR}{LF}{SOH} |
| 82 | Self-test failed | The self-test is run after power is applied. This message is transmitted if it fails. | 82 Self-Test Failed # 01 {CR}{LF}{SOH} 82 Self-Test Failed # 04 {CR}{LF}{SOH} NOTE: The numbers, when converted to binary, show which test(s) failed: Bit 0 = Port A/B Control Test Bit 1 = External RAM Test Bit 2 = Port C Test Bit 3 = Not Used Bit 4 = Microcontroller Test Bit 5 = PROM Test Bit 6 = Not Used Bit 7 = Not Used |
| 83 | Error message | The controller continually checks for errors during operation. This message is transmitted any time a new error occurs. Repeats of the previous error are not retransmitted. | 83 New Error Message = 26 {CR}{LF}{SOH} 83 New Error Message = 3D {CR}{LF}{SOH} NOTE: Refer to the owner's manual for a list of error numbers and descriptions. |
| 86 | Controller reset | Transmitted if the controller resets due to an error | 86 Controller reset. {CR}{LF}{SOH} |
| 90 | X-10 house and unit code received | Transmitted when an X-10 signal consisting of a house code and a unit code is received. | 90 X-10 House/Unit : A 1{CR}{LF}{SOH} 90 X-10 House/Unit : B 9{CR}{LF}{SOH} 90 X-10 House/Unit : G 10{CR}{LF}{SOH} 90 X-10 House/Unit : P 16{CR}{LF}{SOH} NOTE: The units codes are in decimal format, not hex. |

| 91 | X-10 house and function code received | Transmitted when an X-10 signal consisting of a house code and a function code is received. | 91 X-10 House/Func : A On{CR}{LF}{SOH} 91 X-10 House/Func : B Off{CR}{LF}{SOH} 91 X-10 House/Func : C All Lights On{CR}{LF}{SOH} 91 X-10 House/Func : D All Lights Off{CR}{LF}{SOH} 91 X-10 House/Func : E All Units Off{CR}{LF}{SOH} 91 X-10 House/Func : F Bright{CR}{LF}{SOH} 91 X-10 House/Func : G Dim{CR}{LF}{SOH} 91 X-10 House/Func : H Status Request{CR}{LF}{SOH} 91 X-10 House/Func : I Status Is Off{CR}{LF}{SOH} 91 X-10 House/Func : J Status Is On{CR}{LF}{SOH} 91 X-10 House/Func : K Hail Request{CR}{LF}{SOH} 91 X-10 House/Func : L Hail Acknowl.{CR}{LF}{SOH} 91 X-10 House/Func : M Extended Code{CR}{LF}{SOH} 91 X-10 House/Func : N Extended Data{CR}{LF}{SOH} |
| --- | --- | --- | --- |
| 92 | X-10 function code only received | Transmitted when an X-10 signal consisting of only a function code is received. Only the preset dim should fall into this category, as it is the only X-10 signal transmitted without a house code (the house code bits are used to hold the preset dim level instead). However, when HomeVision receives the other function codes, it will sometimes report them in message 92 instead of 91 (due to a bug). You should treat messages 91 and 92 exactly the same. In either case, you need to look at the words to determine what signal was received. | 92 X-10 Func Only : A On{CR}{LF}{SOH} 92 X-10 Func Only : B Off{CR}{LF}{SOH} 92 X-10 Func Only : D All Lights On{CR}{LF}{SOH} 92 X-10 Func Only : C All Lights Off{CR}{LF}{SOH} 92 X-10 Func Only : E All Units Off{CR}{LF}{SOH} 92 X-10 Func Only : Level 00 Preset Dim{CR}{LF}{SOH} 92 X-10 Func Only : Level 01 Preset Dim{CR}{LF}{SOH} 92 X-10 Func Only : Level 0A Preset Dim{CR}{LF}{SOH} 92 X-10 Func Only : Level 0D Preset Dim{CR}{LF}{SOH} 92 X-10 Func Only : Level 10 Preset Dim{CR}{LF}{SOH} 92 X-10 Func Only : Level 1F Preset Dim{CR}{LF}{SOH} NOTE: For the preset dim signals, the number represents the preset dim level (ranging from 0 to 1F), in hex. |
| 93 | X-10 transmission failed | Transmitted when an X-10 transmission fails. This occurs when the controller transmits an X-10 signal the maximum specified number of times, but each attempt fails. | 93 X-10 Transmit Failed (Collision){CR}{LF}{SOH} |
| A1 | Standard format IR signal received | Transmitted when a "standard format" infrared signal is received. This occurs regardless of whether an IR signal that matches the device and key code is defined in the schedule. | A1 Received IR Code = 0A F8 {CR}{LF}{SOH} A1 Received IR Code = BB 16 {CR}{LF}{SOH} NOTE: The first characters (0A and BB in these examples) are the device codes, and the last characters (F8 and 16) are the key codes. All values are in hex. |

| A2 | Defined IR signal received | Transmitted when a "standard format" infrared signal is received that matches the device and key code of a signal defined in the schedule. | A2 Matches IR Signal # = 04 {CR}{LF}{SOH} A2 Matches IR Signal # = 1C {CR}{LF}{SOH} NOTE: The number is the IR signal ID number, in hex. NOTE: This message will always be preceeded by an A1 message reporting the signal's device and key codes, as shown: A1 Received IR Code = 0A F8 {CR}{LF}{SOH} A2 Matches IR Signal # = 04 {CR}{LF}{SOH} |
| --- | --- | --- | --- |
| A3 | Unknown IR signal | Transmitted when an infrared signal is received that doesn't follow the "standard format". This could be another format of remote control signal, or perhaps stray IR emissions from a fluorscent light. | A3 Unknown IR Signal 30 {CR}{LF}{SOH} A3 Unknown IR Signal 31 {CR}{LF}{SOH} A3 Unknown IR Signal 3D {CR}{LF}{SOH} NOTE: The numbers 30, 31, 3D, etc. should be ignored. They are only used by the computer when learning IR signals. |

Note: In many of the examples, a blank space is transmitted immediately before the carriage return.  Pay careful attention to this if you are counting the number of bytes in the message as a validity check.