HomeVision-PC 2.62

Custom Solutions Inc.

Copyright 1996 - 1998

Custom Solutions Inc.

P.O. Box 034206

Indialantic, FL 32903

csi@csi3.com

www.csi3.com

SUMMARY OF CHANGES - 2.62

1.	Added a "Most Recently Used" files list to the "File" menu.  This simplifies opening previously used schedule files.

2.	Added a "New IR Signal" button to the IR Learn screen.  This allows you to create a new infrared signal from the IR Learn screen without having to go to the IR Signal Summary screen.

3.	Added support for lighting “scenes”, which work with some of the newer X-10 modules.

CHANGE DETAILS

X-10 SCENES

Some of the newer X-10 devices, like the LM14A lamp module and the Leviton 6381 wall switches, support “scenes”.  A scene is a group of X-10 modules that can all be set to a predetermined level with a single command.  Scenes require the use of X-10 “extended codes”, which HomeVision can transmit.  HomeVision supports scenes by allowing you to add modules to a scene, remove them from a scene, and execute the scene.  You can have up to four scenes per house code.

There are two steps to using scenes:

1)	Create the scene(s)

2)	Execute a scene

You create scenes by using the X-10 control screen in the PC software.  You can execute a scene from this same screen or from another program (if it supports this capability).

Creating Scenes

To create scenes, open the X-10 control screen in the PC software.  The HomeVision-PC controller must be connected to the PC.  You will use the following two “custom commands” located near the bottom of the custom command list box:

1)	Include in scene # at level XX (of 63)

2)	Remove from scene #

Note that the light levels range from 0 to 63.  This is consistent with the “Set directly to level ##” command that these modules respond to.

As an example, assume you want to create these two scenes using modules A-1, A-2, and A-3:

Scene 1:		A-1 on at level 10

A-2 on at level 15

A-3 on at level 20

Scene 2:		A-1 off (level 0)

A-2 on at level 50

A-3 on at level 63

Here’s how you’d create these scenes:

1)	Select house code “A” and unit code “1”.

2)	Select the custom command “Include in scene # at level XX”.

3)	Enter “1” in the scene number text box.

4)	Enter “10” in the light level text box.

5)	Click the “Perform Command” button.  This will send an X-10 command to module A-1 telling it to include itself in scene 1 at level 10.

6)	Select unit code “2” and level “15” and click the “Perform Command” button to put A-2 in the scene.

7)	Select unit code “3” and level “20” and click the “Perform Command” button to put A-3 in the scene.  Scene 1 is now complete.

8)	Change the scene number to “2”.  Repeat the above steps using the desired levels for scene 2.

The above steps send the necessary commands to the X-10 modules to put them in the two house code “A” scenes.  The modules will retain this information through power failures, so you only need to do this setup once.

If you decide you don’t want a particular module to be in a scene, use the “Remove from scene #” command to remove it.  To test whether the scene works, transmit the “Execute scene #” command, as described in the next section.

Executing Scenes

“Executing” a scene means sending an X-10 signal to command the modules to go to the desired scene.  All modules in the scene should simultaneously go to their preset level.  To execute a scene from the PC, open the X-10 control screen.  Then:

1)	Select the desired house code (the unit code doesn’t matter, as scenes apply to the entire house code).

2)	Select the custom command “Execute scene #”.

3)	Enter the scene number in the scene number text box.

4)	Click the “Perform Command” button.  This will send an X-10 command to the house code and all the lights in the scene should respond.