This appendix details the HomeVision-PC 2.62 serial interface command protocol.  Refer to the file "Serial" for more information.

X-10 COMMANDS [Command types P (20) and X (28)]

¥	Module ID number (2 bytes).  Module ID numbers start with zero representing A1 and progress upwards through FFh (255d), which represents P16.  Values are entered in hex format, as shown in the following table:

|  |  | HOUSE CODE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F | G | H | I | J | K | L | M | N | O | P |
|  | 1 | 00 | 10 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | A0 | B0 | C0 | D0 | E0 | F0 |
|  | 2 | 01 | 11 | 21 | 31 | 41 | 51 | 61 | 71 | 81 | 91 | A1 | B1 | C1 | D1 | E1 | F1 |
|  | 3 | 02 | 12 | 22 | 32 | 42 | 52 | 62 | 72 | 82 | 92 | A2 | B2 | C2 | D2 | E2 | F2 |
| U | 4 | 03 | 13 | 23 | 33 | 43 | 53 | 63 | 73 | 83 | 93 | A3 | B3 | C3 | D3 | E3 | F3 |
| N | 5 | 04 | 14 | 24 | 34 | 44 | 54 | 64 | 74 | 84 | 94 | A4 | B4 | C4 | D4 | E4 | F4 |
| I | 6 | 05 | 15 | 25 | 35 | 45 | 55 | 65 | 75 | 85 | 95 | A5 | B5 | C5 | D5 | E5 | F5 |
| T | 7 | 06 | 16 | 26 | 36 | 46 | 56 | 66 | 76 | 86 | 96 | A6 | B6 | C6 | D6 | E6 | F6 |
|  | 8 | 07 | 17 | 27 | 37 | 47 | 57 | 67 | 77 | 87 | 97 | A7 | B7 | C7 | D7 | E7 | F7 |
| C | 9 | 08 | 18 | 28 | 38 | 48 | 58 | 68 | 78 | 88 | 98 | A8 | B8 | C8 | D8 | E8 | F8 |
| O | 10 | 09 | 19 | 29 | 39 | 49 | 59 | 69 | 79 | 89 | 99 | A9 | B9 | C9 | D9 | E9 | F9 |
| D | 11 | 0A | 1A | 2A | 3A | 4A | 5A | 6A | 7A | 8A | 9A | AA | BA | CA | DA | EA | FA |
| E | 12 | 0B | 1B | 2B | 3B | 4B | 5B | 6B | 7B | 8B | 9B | AB | BB | CB | DB | EB | FB |
|  | 13 | 0C | 1C | 2C | 3C | 4C | 5C | 6C | 7C | 8C | 9C | AC | BC | CC | DC | EC | FC |
|  | 14 | 0D | 1D | 2D | 3D | 4D | 5D | 6D | 7D | 8D | 9D | AD | BD | CD | DD | ED | FD |
|  | 15 | 0E | 1E | 2E | 3E | 4E | 5E | 6E | 7E | 8E | 9E | AE | BE | CE | DE | EE | FE |
|  | 16 | 0F | 1F | 2F | 3F | 4F | 5F | 6F | 7F | 8F | 9F | AF | BF | CF | DF | EF | FF |

¥	Command number (2 bytes).  Commands without a Ò##Ó value are 4-byte commands (type X).  Commands with a Ò##Ó value are 6-byte commands (type P).

Cmd # 00 (type X) = On

Cmd # 01 (type X) = Force On

Cmd # 02 (type X) = Off

Cmd # 03 (type X) = Force Off

Cmd # 04 (type X) = Set State To Neutral

Cmd # 05 (type X) = Dim

Cmd # 06 (type X) = Force Dim

Cmd # 07 (type P) = Dim ## Times

Cmd # 08 (type P) = Dim ## Times Slow

Cmd # 09 (type P) = Force Dim ## Times

Cmd # 0A (type P) = Force Dim ## Times Slow

Cmd # 0B (type X) = Brighten

Cmd # 0C (type X) = Force Brighten

Cmd # 0D (type P) = Brighten ## Times

Cmd # 0E (type P) = Brighten ## Times Slow

Cmd # 0F (type P) = Force Brighten ## Times

Cmd # 10 (type P) = Force Brighten ## Times Slow

Cmd # 11 (type P) = Set To Level ##

Cmd # 12 (type P) = Set To Level ## Slow

Cmd # 13 (type P) = Force To Level ##

Cmd # 14 (type P) = Force To Level ## Slow

Cmd # 15 (type X) = Send House/Unit Code Only

Cmd # 16 (type P) = Send House/Function Code ## Only

## values are:

On 				= 02

Off 				= 0A

Brighten 			= 06

Dim 				= 0E

All Lights On 		= 04

All Units Off 		= 0C

All Lights Off		= 00

Status Request		= 09

Status Is On 		= 07

Status Is Off 		= 01

Hail Request 		= 0D

Hail Acknowledge	= 05

NOTE: These commands do not send a unit code.  When entering a module ID value, you can use the ID number of any of the 16 modules with the desired house code.  For example, module ID numbers 00 through 0Fh (15d) all will send house code A.

Cmd # 17 (type X) = Toggle Module

Cmd # 18 (type X) = Enable Module

Cmd # 19 (type X) = Disable Module

Cmd # 1A (type X) = Set State To On

Cmd # 1B (type X) = Set State To Off

Cmd # 1C (type X) = Transmit Module Status Signal

Cmd # 1D (type P) = Transmit Preset Dim To Level ##

Cmd # 1E (type X) = Read Current State

NOTE: This command will cause the controller to transmit the current state over the serial interface.  The value should be converted to 8 bits and interpreted as follows:

bit    0 :  0 = Disabled

1 = Enabled

bits 1-2 : 00 = Off

01 = On

10 = Neutral

bits 3-7 : current level (0-16)

Note that the bits are ordered as follows: 76543210

Example: A light on at level 15 (decimal) and enabled will have this value:

01111011

This is the value 7Bh, and 7B will be transmitted.

Cmd # 1F (type X) = Refresh Module

Cmd # 20 (type X) = Refresh House Code

Cmd # 21 (type P) = Set PCS Light To Level ##

Cmd # 22 (type X) = Micro-Brighten PCS Light 1 Level (1/2%)

Cmd # 23 (type X) = Micro-Dim PCS Light 1 Level (1/2%)

Cmd # 3B (type g) = Transmit Extended Data Bytes: DATA  &  COMMAND

Cmd # 3C (type g) = Transmit Extended Code Bytes: DATA  &  COMMAND

For command type P (6-byte X-10 commands), 2 additional bytes are also required:

¥	2 bytes for Ò##Ó data value required for the command (shown above).

For command type g (8-byte X-10 commands), 4 additional bytes are also required:

¥	2 bytes for  DATA  value required for the command (shown above).

¥	2 bytes for  COMMAND  value required for the command (shown above).

EXAMPLES:

¥	To send an ON command to module A1 (ID # 00), use the FORCE ON command (01):

,X0001

¥	To send an OFF command to module A2 (ID # 01), use the FORCE OFF command (03):

,X0103

¥	To send 5 DIM commands to module B2 (ID # 11), use command 7:

,P110705

¥	To send house and unit codes for module B3 (ID # 12), use command 15:

,X1215

¥	To send an ALL LIGHTS ON command to house code A, use command 16 with any module ID number from 00h to 0Fh:

,P001604    or    ,P0A1604    or    ,P0F1604

¥	To send an BRIGHT command to house code P, use command 16 with any module ID number from F0h to FFh:

,PF01606    or    ,P091606    or    ,P0C1606

INPUT PORT COMMANDS [Command type : (0A)]

¥	Input port ID number (2 bytes).

¥	Command number (2 bytes):

Cmd # 00 = Not Used

Cmd # 01 = Not Used

Cmd # 02 = Not Used

Cmd # 03 (type :) = Disable Input Port

Cmd # 04 (type :) = Enable Input Port

Cmd # 05 (type :) = Read Input Port State

When this command is issued over the serial interface, the controller will transmit the current state (2 LSBs) over the serial interface.  The value is interpreted as follows:

00 = port low and disabled

01 = port low and enabled

02 = port high and disabled

03 = port high and enabled

EXAMPLES:

¥	To disable input port 07, use command 03:

,:0703

¥	To transmit the current state of input port 07 over the serial interface, use command 05:

,:0705

OUTPUT PORT COMMANDS [Command types 3 (03), M (1D), and Z (2A)]

¥	Output port ID number (2 bytes).

¥	Command number (2 bytes).  Commands without Ò##Ó values are 4-byte commands (type 3).  Commands with a Ò##Ó value are 6-byte commands (type M).  Commands with a Ò#####Ó value are 8-byte commands (type Z).

Cmd # 00 (type 3) = Set Output Port Low

Cmd # 01 (type 3) = Set Output Port High

Cmd # 02 (type 3) = Toggle Output Port

Cmd # 03 (type 3) = Disable Output Port

Cmd # 04 (type 3) = Enable Output Port

Cmd # 05 (type 3) = Read Output Port State

When this command is issued over the serial interface, the controller will transmit the current state (2 LSBs) over the serial interface. The value is interpreted as follows:

00 = port low and disabled

01 = port low and enabled

02 = port high and disabled

03 = port high and enabled

Cmd # 06 = Not used

Cmd # 07 (type Z) = Pulse Output Port High For ##.### Seconds

Convert the time to milliseconds (0 - 65,535) and enter in the next 4 bytes, MSB first

Cmd # 08 (type Z) = Pulse Output Port Low For ##.### Seconds

Convert the time to milliseconds (0 - 65,535) and enter in the next 4 bytes, MSB first

Cmd # 09 (type M) = Set 8-Bit Port To ##

The port to write to is determined by the Output Port ID Number.  00 is for Port A, 01 is for Port C.  The ## value is the binary values for the 8 lines of the selected port.  Enter the ## value in the next 2 bytes.

For command type M (6-byte output port commands), 2 data bytes are also required:

¥	Enter Ò##Ó data value required for command 09.

For command type Z (8-byte output port commands), 4 data bytes are also required:

¥	Enter milliseconds value in 4 bytes, with the most significant byte first

EXAMPLES:

¥	To set output port 1 low, use 4-byte command 00:

,30100

¥	To set output port 0 high, use 4-byte command 01:

,30001

¥	To pulse output port 3 high for 5 milliseconds, use 8-byte command 07:

,303070005

To pulse it high for 1 second, first convert it to milliseconds.  1 second is 1000 milliseconds.  Then convert 1000 (decimal) to hex: 03E8.  The command is then:

,3030703E8

¥	LetÕs say we want to set all 8 lines of output port A simultaneously as follows:

Line A-0 = High (1)

Line A-1 = High (1)

Line A-2 = Low (0)

Line A-3 = High (1)

Line A-4 = Low (0)

Line A-5 = Low (0)

Line A-6 = Low (0)

Line A-7 = High (1)

First, we must determine the value which corresponds to the state of the 8 lines indicated above.  The binary representation is as follows:

10001011

Note that the highest port (A-7) is first (the most significant bit) and the lowest port (A-0) is last (the least significant bit).  This 8-bit binary number must then be converted to hex.  The result is 8B (139d).

Thus, the full command is:

,M00098B

INFRARED SIGNAL COMMANDS [Command type ; (0B) and < (0C)]

¥	IR signal ID number (2 bytes).

¥	Command number (2 bytes):

Cmd # 00 (type ;) = Transmit IR Signal Once

Cmd # 01 = Not used

Cmd # 02 (type ;) = Disable IR Signal Receive Mode

Cmd # 03 (type ;) = Enable IR Signal Receive Mode

Cmd # 04 (type ;) = Disable IR Signal Transmit Mode

Cmd # 05 (type ;) = Enable IR Signal Transmit Mode

¥	To transmit an IR signal multiple times, use command type <.  Enter the signal ID # followed by the number of times to transmit it.

EXAMPLES:

¥	To transmit IR signal 20d (14h), use command 00:

,;1400

¥	To transmit IR signal 20d (14h) 7 times, use command type <:

,<1407

¥	To disable reception of IR signal 21d (15h), use command 02:

,;1502

CONTROLLER COMMANDS [Command type G (17)]

¥	Command number (2 bytes):

Cmd # 00 (type G) = Check Communications

This command does nothing but respond over the serial interface

Cmd # 01 (type G) = Halt Controller

Cmd # 02 (type G) = Resume Controller

Cmd # 1B (type G) = Turn User LED On

Cmd # 1C (type G) = Turn User LED Off

Cmd # 1D (type G) = Read Error Message

Cmd # 1F (type G) = Clear Error Message

Cmd # 21 (type G) = Transmit Controller Status

Response data is a value between 0 and 255.  When converted to binary, the meaning of each bit can be found as follows:

bit 0 : 0 = Running, 1 = Halted

bit 1 : 0 = OK, 1 = controller error

bit 2 : 0 = TW-523 OK, 1 = no TW-523

bit 3 : 0 = using RAM config, 1 = using default config

bit 4 : 0 = self test passed, 1 = self test failed

bit 5 : not used

bit 6 : 0 = RAM checksum OK, 1 = RAM checksum error

bit 7 : not used

Cmd # 24 (type G) = Execute Self-Test

Cmd # 27 (type G) = Set Baud Rate to 1200

Cmd # 28 (type G) = Set Baud Rate to 2400

Cmd # 29 (type G) = Set Baud Rate to 4800

Cmd # 2A (type G) = Set Baud Rate to 9600

Cmd # 2B (type G) = Set Baud Rate to 19200

NOTE: All baud rate changes take effect after the current serial transmissions are completed.

Cmd # 46 (type G) = Enable Logging of Received X-10 Signals

Cmd # 47 (type G) = Disable Logging of Received X-10 Signals

Cmd # 48 (type G) = Enable Logging of Input Ports

Cmd # 49 (type G) = Disable Logging of Input Ports

EXAMPLES:

¥	To turn the user LED on, use command 1B:

,G1B

¥	To set the controller's baud rate to 9600, use command 2A:

,G2A

VIDEO COMMANDS - 4 BYTES [Command type L (1C)]

¥	Command number (2 bytes):

Cmd # 00 (type L) = Not used

Cmd # 01 (type L) = Set Cursor Row To ##

Cmd # 02 (type L) = Set Cursor Column To ##

Cmd # 03 (type L) = Do Line Feed And Carriage Return

Cmd # 04 (type L) = Clear Screen

Cmd # 05 (type L) = Turn On Screen Display ##

## value meaning:

00 = Off

01 = On

Cmd # 06 (type L) = Set OSD To ## Video Background

## value meaning:

00 = External

01 = Internal

Cmd # 07 (type L) = ## Video Mode

## value meaning:

00 = Start External

01 = Start Internal

02 = Stop Video

Cmd # 08 (type L) = Start Video Menu System

Cmd # 09 (type L) = Display Video Screen ##

Cmd # 0A (type L) = Set Character Blink ##

## value meaning:

00 = Off

01 = On

Cmd # 0B (type L) = Set Row ______ Character Size To ______

## value meaning:

Upper 4 bits contains character size

Lower 4 bits contains row number

Cmd # 0C (type L) = Display Character Number ## On Screen

This command is used over the serial interface to display characters on the screen.  Each command sends a single character.  ## are the two hex digits representing the ASCII value of the desired character.  For example:

## Value	    Character Displayed

30			0

31			1

39			9

3A			:

41			A

5A			Z

61			a

7A			z

Note that: The ASCII values for the numbers and letters can be found in the appendix to the userÕs manual.  However, many characters can be displayed that do not have a similar ASCII equivalent.  A table is provided following the video command examples that shows the ASCII values for these characters.

Cmd # 0F (type L) = Display Text String Number ## On Screen

Cmd # 10 (type L) = Set Video Switch To ##

## value meaning:

00 = External Video Position

01 = OSD Video Position

Cmd # 11 (type L) = Initialize OSD Circuits

Cmd # 12 (type L) = Set Character Blink Ratio To ##

## value meaning:

00 = Off

01 = 1:3

02 = 3:1

03 = 1:1

Cmd # 13 (type L) = Set Character Background Style To ##

## value meaning:

00 = None

01 = Outline

02 = Square

03 = Solid

Cmd # 14 (type L) = Select Current Screen Background To ##

## value meaning:

00 = Black

01 = Blue

02 = Green

03 = Cyan

04 = Red

05 = Magenta

06 = Yellow

07 = White

Cmd # 15 (type L) = Set Vertical Screen Offset To ##

Cmd # 16 (type L) = Set Horizontal Screen Offset To ##

Cmd # 1B (type L) = Set Default Screen Background To ##

## value meaning:

00 = Black

01 = Blue

02 = Green

03 = Cyan

04 = Red

05 = Magenta

06 = Yellow

07 = White

¥	2 bytes for ## value (as described in above command list).  For commands without an ## value, enter 2 zeros.

EXAMPLES:

¥	To set the cursor to row 2, use command 01:

,L0102

¥	To clear the screen, use command 04:

,L0400

¥	To start the internal video mode, use command 07:

,L0701

¥	To stop the video mode, use command 07:

,L0702

¥	To start the video menu system, use command 08:

,L0800

¥	To display video screen 04, use command 09:

,L0904

¥	To set row 1 size to 2, use command 0B:

,L0B21

¥	To set row 1 size to 3, use command 0B:

,L0B31

¥	To set row 10d (Ah) size to 4, use command 0B:

,L0B4A

¥	To display the number Ò5Ó, use command 0C with an ASCII value of 3555:

,L0C35

¥	To display the letter ÒAÓ, use command 0C with an ASCII value of 41:

,L0C41

¥	To display the letter ÒaÓ, use command 0C with an ASCII value of 61:

,L0C61

¥	To display a comma, use command 0C with an ASCII value of 2C:

,L0C2C

¥	To display the telephone symbol, use command 0C with an ASCII value of 26:

,L0C26

¥	To set the video switch to the on-screen display position, use command 10:

,L1001

¥	To set the current screen background color to red, use command 14:

,L1404

VIDEO CHARACTERS AND THEIR ASCII VALUES

| DISPLAYED CHARACTER | ASCII VALUE | DESCRIPTION |
| --- | --- | --- |
| . | 2E | Period |
| , | 2C | Comma |
| : | 3A | Colon |
| ; | 3B | Semicolon |
| ? | 3F | Question Mark |
| - | 2D | Dash |
| / | 2F | Slash |
| See note | 20 | Blank Space |
| < | 3C | Left Angled Bracket |
| > | 3E | Right Angled Bracket |
| See note | 60 | Snowman |
| See note | 21 | Left Arrow |
| See note | 40 | Up Arrow |
| See note | 23 | Right Arrow |
| See note | 24 | Down Arrow |
| See note | 25 | Small Solid Square |
| See note | 7C | Large Solid Square |
| . | 5E | Point (like a period, but shifted up and left slightly) |
| x | 2A | Multiply Sign (same as small letter x, but shifted up slightly) |
| See note | 26 | Telephone |
| See note | 28 | Man |
| See note | 29 | Woman |
| See note | 5B | Reverse Symbol |
| See note | 5D | Forward Symbol |
| See note | 5C | Sun |
| See note | 22 | Your guess is as good as ours! |
| See note | 3D | Umbrella |
| See note | 27 | Musical Note |
| See note | 5F | No Display |

NOTE: These characters could not be drawn in this table.  YouÕll have to display then and see for yourself what they look like.

VIDEO COMMANDS - 15 BYTES [Command type d (34)]

This command is used to write up to 14 characters to the video screen.

¥	1 byte indicating the number of characters to display.  Valid values are 1 - 14.  The byte value must be in hex format ("1" for 1 character, "9" for 9 characters, "A" for 10 characters, "E" for 14 characters, etc.).

¥	14 bytes, 1 for each character to display.  You must transmit all 14 bytes, even if you specify a smaller number to display.  Each byte should be the actual character to display ("1" for the number 1, "A" for the capital letter A, h for the small letter h, etc.)

EXAMPLES:

¥	To write (display) "Happy Birthday", send this command:

,dEHappyBirthday

¥	To write (display) "Hello", send this command:

,d5HelloXXXXXXXXX

Note that the last 9 characters can be anything, as they aren't used since the command is for only 5 bytes.

SYSTEM VARIABLE COMMANDS [Command type ^ (5E)]

¥	Command number (2 bytes):

Cmd # 00 (type ^) = Set System Variable XX Equal To Constant Value ##

Cmd # 01 (type ^) = Not used

Cmd # 02 (type ^) = Not used

Cmd # 03 (type ^) = Read Value Of System Variable XX Over Serial Interface

¥	System variable ID number (2 bytes).  Each system variable has a unique ID number, as shown below:

00 = Current inside temperature  (see note 1)

01 = Today's low inside temperature  (see note 1)

02 = Today's high inside temperature  (see note 1)

03 = Current outside temperature  (see note 1)

04 = Today's low outside temperature  (see note 1)

05 = Today's high outside temperature  (see note 1)

NOTE 1: The temperatures can range from -99 to +154 degrees in increments of 1 degree.  Temperatures are stored as actual temperature plus 100.  HomeVision-PC subtracts 100, then displays it, with two exceptions.  On the weather conditions video screen, the value of 0 is displayed as -99 degrees instead of -100.  A value of 255 is displayed as blank spaces instead of a number (this is so that temperatures that aren't available can be left blank).  Here are some examples:

STORED VALUE    DISPLAYED VALUE

0	-99

1	-99

2	-98

99	 -1

100	0

101	1

175	75

254	154

255

06 = Current inside relative humidity  (see note 2)

07 = Today's low inside relative humidity  (see note 2)

08 = Today's high inside relative humidity  (see note 2)

09 = Current outside relative humidity  (see note 2)

0A = Today's low outside relative humidity  (see note 2)

0B = Today's high outside relative humidity  (see note 2)

NOTE 2: Relative humidities can range from 0 to 100 in increments of 1%.  A value of 255 is displayed as blank spaces.

0C = Current barometric pressure - inches  (see note 3)

0D = Current barometric pressure - hundredths  (see note 3)

NOTE 3: Barometric pressure requires two variables for storage.  Each variable can range from 0 to 99 in increments of 1%.  All other values are displayed as blank spaces.

0E =	Barometric Pressure Direction (0 = steady, 1 = falling, 2 = rising, anything else = blank spaces are displayed)

0F = Wind direction digit 1  (see note 4)

10 = Wind direction digits 2-3  (see note 4)

NOTE 4: Wind direction (in degrees from 0 to 359) requires two variables for storage.  The first variable holds the first digit, which can range from 0 to 3.  The second variable holds the second and third digits, which can range from 00 to 99 (in hex format).  If the first variable is >3, then blanks will be displayed on the weather conditions TV screen.

11 = Current wind speed  (see note 5)

12 = Today's low wind speed  (see note 5)

13 = Today's high wind speed  (see note 5)

NOTE 5: Wind speed can range from 0 to 254 in increments of 1.  A value of 255 is displayed as blank spaces.

14 = Today's rainfall - inches  (see note 6)

15 = Today's rainfall - hundredths  (see note 6)

16 = Rainfall this week - inches  (see note 6)

17 = Rainfall this week - hundredths  (see note 6)

18 = Rainfall this month - inches  (see note 6)

19 = Rainfall this month - hundredths  (see note 6)

1A = Rainfall this year - inches  (see note 6)

1B = Rainfall this year - hundredths  (see note 6)

NOTE 6: Four different rainfall amounts can be tracked, with each requiring two variables for storage.  The "hundredths" values can range from 0 to 99 in hex format.  Only for the variable "Todays rainfall - inches", a value of >99 is displayed as blank spaces instead of the digits.  The other rainfall "inches" amounts are not included on the weather conditions screen and can range from 0 to 255 inches.

¥	2 bytes for Ò##Ó value required for the command:

-	For command 00, enter the constant value in hex format (from 00 to FF).

-	For commands 01 and 02, enter the variable ID # you want to use (in hex format).  Note that this refers to the variables you created in your schedule.

-	For command 03, enter two zeros (00).

EXAMPLE:

¥	To set the system variable "Current outside temperature" to indicate 65 degrees (41 hex), use command 00:

,^000341

¥	To set the current barometric pressure to 30.08 inches and falling, use these commands:

,^000C1E					{30 inches = 1E hex}

,^000D08					{8 hundredths = 08 hex}

,^000E01					{01 = falling}

¥	To set the current wind direction to due south (180 degrees), use these commands:

,^000F01					{"1" first digit = 01 hex}

,^001050					{"80" last 2 digits = 50 hex}

¥	To set the total rainfall this year to 12.90 inches, use these commands:

,^001A0C					{12 inches = 0C hex}

,^001B5A					{90 hundredths = 5A hex}

SIMULATE INFRARED REMOTE KEYPRESS [Command type J (4A)]

These commands simulate pressing an IR remote button to control the video menu system.  When you send a command, the controller acts on it as if it received the corresponding signal from an IR remote.  This feature is provided to allow voice control of the video menu system.  You can setup voice recognition software to send the serial command to HomeVision-PC when you speak the corresponding word(s).

This command is sent as a single character representing each of the 16 remote buttons used for the video menu system:

CHARACTER      REMOTE BUTTON

0				0

1				1

2				2

3				3

4				4

5				5

6				6

7				7

8				8

9				9

A				Enter

B				Channel Up

C				Channel Down

D				Volume Up

E				Volume Down

F				Cancel

EXAMPLE:

¥	When the video system is off, the Channel Up button starts it.  You can simulate this with this command:

,JB

¥	From the video menu screen, the number 1 selects the X-10 menu entry.  Simulate this by:

,J1

¥	From the X-10 signal list, the number 1 selects X-10 signal A-1.  Simulate this by:

,J1

¥	From the X-10 signal list, the Channel Down button exits the menu system.  Simulate this by:

,JC