HomeVision-PC 2.62

Custom Solutions Inc.

Copyright 1996 - 1998

Custom Solutions, Inc.

P.O. Box 034206

Indialantic, FL 32903

csi@csi3.com

VERSION HISTORY OVERVIEW

2.62 (11 October 1998)

New software which:

-	Added a "Most Recently Used" files list to the "File" menu.  This simplifies opening previously used schedule files.

-	Added a "New IR Signal" button to the IR Learn screen.  This allows you to create a new infrared signal from the IR Learn screen without having to go to the IR Signal Summary screen.

-	Added support for lighting “scenes”, which work with some of the newer X-10 modules.

Refer to the "NewInfo" file for details on these latest changes.

2.61 (1 March 1998)

New software which:

-	Added X-10 commands to set the new X-10 lamp and applicance modules (LM14A and AM14A) and Leviton 6381 modules directly to a level without going to full brightness first.  This feature requires the version 2.6 or later PROM.

-	Made several changes to improve performance for international users.  These improvements function even when using older version 2.5 PROMs.

2.5 (3 September 1997)

Initial release.

CHANGES IN PREVIOUS VERSIONS

VERSION 2.61

X-10 MODULE DIRECT LEVEL COMMANDS

The new Leviton 6381 switches and the X-10 LM14A and AM14A modules have several new features.  One of these is that they can be directly set to a level without going on to full brightness first.  This is sometimes called "preset dimming", although this is not technically correct.

HomeVision-PC version 2.61 can control these lights using X-10's "extended codes".  The "extended codes" follow a different format than the older extended codes.  You should also note that these are different than the "preset dim" commands used by the PCS brand lighting modules.

The modules can be set to any of 64 levels (0 to 63).  Sending a level 63 command immediately turns the light on to full brightness.  Any other level command causes the light to slowly change to the desired level over several seconds.  If the module is off, it does not go to full brightness first (a big improvement over prior models!).  In our testing, it appears that levels 62 and 63 are the same (full on).  However, the level 62 command causes the light to slowly brighten, while the level 63 command sets it immediately to full on.  Level 0 is the same as sending a standard OFF command.  These modules also respond to all the standard X-10 commands (ON, OFF, BRIGHTEN, DIM, etc.).

The new command has the following name:

X-10 Device Directly To Level ##

You can transmit this command from the X-10 Control Screen in the PC software.  It is located near the bottom of the "Custom Command" section.

You should note that these modules may be incompatible with certain X-10 "repeaters".  If you have trouble with them, contact the manufacturer.

These modules have a variety of other capabilities that use extended codes, including transmitting their status.  Unfortunately, the TW-523 module cannot receive these extended X-10 codes, so HomeVision cannot use them.  However, HomeVision can transmit all these codes from the X-10 Control Screen.  These commands are located near the bottom of the "Custom Command" section.  The commands are:

Transmit extended codes ## and ##

Transmit extended data ## and ##

When you select one of these commands, you will have to enter the command and data values (represented by ##).  You must refer to the X-10 documentation which details the available commands, their values, and how to use them.