HomeVision-PC 2.62

Custom Solutions Inc.

Copyright 1996 - 1998

Custom Solutions Inc.

P.O. Box 034206

Indialantic, FL 32903

csi@csi3.com

www.csi3.com

DOCUMENT PURPOSE

HomeVision-PC can be controlled by commands over the serial interface.  It can also report events over the serial interface.  Once you understand the protocol, you can create your own programs to control HomeVision-PC and/or respond to messages from HomeVision-PC.  This file provides details of the interface protocol.

SERIAL INTERFACE COMMANDS TO HOMEVISION-PC

There are many commands that can be transmitted to the controller over the serial interface.  When received by the controller, it executes the command and reports the results back over the serial interface.

COMMAND STRUCTURE

Commands issued to the controller must follow a specific format in order to be recognized.  Although commands differ in the amount of data required, they all follow the same basic format:

Header Character (a comma)

Command Character

<Command Data>

Terminator Character (a carriage return)

The meanings of each portion of the command are provided below:

¥	Header Character:  A single character (a comma) that tells the controller that a command will follow.  All commands must start with this header character.

¥	Command Character:  A single, unique character that identifies the type of command.  There are different characters for X-10 commands, infrared commands, etc.  The command character for each command type is given in Table 1.  Note that the command character is case-sensitive.

¥	<Command Data>:  Consists of 0 or more bytes of data that define what the command must do.  For example, when commanding the controller to transmit an X-10 signal, you must specify the house/unit code (such as A1, C7, etc.) and the function (On, Off, Dim, etc.).  These are supplied in the ÒDataÓ field.  The brackets <> indicate that the data length varies with different commands.  Some commands require no data at all.  For these, the ÒDataÓ field is skipped.  The total command length (in bytes) and any requried data bytes are provided in Table 1.

¥	Terminator:  A single character (a carriage return) that completes the command and tells the controller to execute it.

COMMAND RESPONSE

If the controller successfully performs the command, it reports that the command was completed by transmitting the following message over the serial interface:

Done(Carriage Return)(Line Feed)(SOH)

Only the word ÒDoneÓ will appear on a terminal emulator.  YouÕll notice that three bytes of information are transmitted after ÒDoneÓ.  The carriage return and line feed move the cursor to the start of the next row.  SOH is the ASCII character with a binary value of 1.  This byte is transmitted by the controller to make it easier for a PC program to determine when the command response is complete.  When you see this value, you know that the response is complete.  The only other time the controller would transmit this value is if your schedule includes a command to transmit the binary value of 1.

COMMAND GENERATION

There are several different ways to generate and send a serial command to the controller:

1)	Use a communications or terminal emulation program (such as the ÒTerminalÓ program supplied with Windows).  Almost all of these programs provide the capability to type data on the keyboard and have the character sent directly out the serial port.  The programs also display any data received from the controller.  All you have to do is configure the program (selecting the appropriate comm port, baud rate, etc.), then start typing.

Most programs send the data immediately after the key is pressed.  Pressing the back space or delete keys cannot undo what has already been transmitted to the controller.  If you make a mistake while transmitting data to the controller, it might execute an unplanned command.  Be sure to read and understand the section on aborting commands before you try to send commands from a terminal program.

2)	Use the controller command interface built into the PC software (located under the ÒControlÓ menu).  Unlike terminal emulators, this command screen doesnÕt transmit any data until the carriage return key is pressed.  When it is pressed, the entire line is transmitted with a carriage return at the end.  This allows you to see the entire command and verify itÕs right before transmitting it.  You should note several things about the screen:

-	The carriage return will not appear in the command line, but it will be automatically transmitted at the end of the line.  This carriage return is interpreted by the controller as the command terminator.

-	The command line is not cleared after transmitting it.  If you press the carriage return again, the same command will be transmitted again.

-	The window will display any data received from the controller.  The CLEAR button will clear this window.

3)	Send serial data from another application.  Any program that can transmit data over the serial interface can transmit commands to the controller.

COMMAND PROCESSING

This section describes how the controller decodes the incoming serial data to identify the command.

Whenever the controller receives serial data, it checks to see if it follows the HomeVision-PC command format.  After a complete and valid command is received, it is executed.  Any invalid data will terminate the receive cycle (all previously received data will be ignored) and the controller will start over waiting for the next command.  The receive cycle steps through the following states:

1)	The controller waits for a header character (a comma) to be received.  Any data that is not a comma is ignored.  Only when a valid header is received will it go to State 2.

2)	The controller now expects to receive a command character.  If the character immediately following the header is not a valid command character, the receive cycle is terminated and the controller returns to State 1 to wait for another header character.  If it is a valid command character, it moves to State 3.

3)	The controller is now expecting to receive a specific number of data bytes (which can be different for each command).  If the command does not require any data, it immediately goes to State 4.  Command characters that require data have a specific data length weÕll call ÒLÓ.  The next L bytes received are stored in an internal buffer.  After the last byte (byte number L) is received, the controller moves to State 4.

4)	The controller now expects to receive the terminator code (a carriage return).  If any other character is received instead, the receive cycle is terminated and the controller returns to State 1 to await another header character.  The command is ignored.  If the proper terminator code is received, the controller moves to State 5.

5)	The controller acknowledges that a valid command was received by transmitting the following response over the serial interface:

## Cmd:

In the response, the Ò##Ó will be replaced by the Command ID Number of the received command (as shown in Table 1).  There is no carriage return or line feed at the end of the line.  The controller then moves to State 6.

6)	The controller now executes the command based on the command character and the associated data.  If the command requires a data response from the controller, it will be transmitted here.  After completing the command, the controller moves to State 7.

7)	The controller reports that the command was completed by transmitting the following message over the serial interface, as discussed previously:

Done(Carriage Return)(Line Feed)(SOH)

The command process is now complete and the controller returns to State 1 to wait for another command.

TO ABORT COMMANDS ENTERED IN ERROR

ItÕs easy to make a mistake when entering data from a keyboard.  If youÕre using a terminal emulation program that transmits each character as the key is pressed, this could cause unexpected results.  If you start entering a command and realize youÕve made a mistake, the command can always be aborted.  How this is done depends on where in the command entry sequence you made the mistake.

¥	If you enter the header character (a comma) by mistake, press any key that is not a valid command character.  The space bar is probably the easiest choice.  This immediately aborts the receive process.

¥	If you enter the header character and then a wrong command character, the controller will be expecting either data or the terminator.  The only way to abort is to first enter the required amount of data (or more), then enter an invalid terminator character.  Since different commands require different amounts of data, you may not be sure how many times to enter a keystroke.  Some commands expect 16 bytes of data, so you need to press a key (any key except the carriage return) at least 16 times to fill up the data buffer.  After the controller has received the amount of data it expects, it will be looking for the carriage return as a terminator.  Pressing any other key at this point will abort the receive process.

The safest way to get out of this mess is to press any key EXCEPT FOR THE CARRIAGE RETURN 17 times.  This will always abort the receive process regardless of which command character was entered (it does this by causing an incorrect terminator).  However, if you know how many data bytes are required for the command character you entered inadvertantly, you may not really need 17 keypresses.  Simply enter 1 more byte that the required data length.  For example, if the command requires 4 bytes, enter 5 keypresses.  This will fill up the data buffer, and then abort when an invalid terminator character is received.  Again, you can use any key except for the carriage return (the carriage return is the expected terminator).

¥	If youÕve already entered the header, command character , and its data, the controller is expecting the terminator.  Press any key except for the carriage return and the receive process will be aborted.

The key to this process is to make sure not to press the carriage return (the terminator) when the controller is waiting for it.

CONTROLLER COMMAND FORMATS

Table 1 lists all of the controller command types and the format of the data in each.  The meanings of the columns are explained below:

¥	ÒCOMMAND DESCRIPTIONÓ

This is a brief description of the command type.  YouÕll notice several things about the available commands:

-	Most command types are for specific objects, such as X-10 modules or input ports.  Each object type has one or more commands to control them.

-	Some object types have multiple commands, each requiring different amounts of data.  For example, there is one command type for X-10 commands which require 4 bytes of data.  There is a different command type for X-10 commands which require 6 bytes of data.

¥	ÒCOMMAND CHARACTER / ASCII VALUEÓ

The character listed at the left of this column is the Command Character.  If you are entering commands from a terminal emulator program, this is the key you would press immediately after the header character.  The three digits at the right of the column show the ASCII value of the character.  The ÒhÓ indicates the value is shown in hex format.  If you are transmitting binary data from a PC program, this is the value to transmit (itÕs the same as transmitting the character).

There are other command characters not listed in Table 1.  They are used to load schedules and read and write internal memory.  Transmiting these commands could corrupt the schedule, requiring reloading.  Therefore, command characters not listed in the table should never be sent.

¥	ÒCOMMAND LENÓ

This is the command data length.  This many bytes (characters) must be provided in the <data> field of the command.

¥	ÒCOMMAND DATAÓ

This column describes the order of the data in the <data> field (the actual data to be entered is described in the Appendix).  The numbers such as Ò1/2Ó indicate the order in which the data is entered.  The data is usually entered in pairs of characters.  Ò1/2Ó indicates that characters 1 and 2 together make up the first value.

The data values always range from 0 to 255.  Each value is entered as two characters in hex format.  Here are some examples:

To Enter This Decimal Value    Type These Characters

0									00

1									01

9									09

10									0A

15									0F

16									10

17									11

32									20

48									30

64									40

80									50

255									FF

Notice that you must always enter two characters, using a leading zero where necessary.  The characters A, B, C, D, E, and F needed to represent hex values are case-sensitive and must be capitalized.

Several points should be noted about the command data:

-	Many command types require an object ID number.  This is the same number listed on the object summary screens for each object.  It ranges from 0 to 255.  This specifies the object that the command will take action on.

For some commands, this field is not used, but you must still enter a value.  You can simply enter zero (actually two 0Õs).

For some commands, this field is used to provide data other than an object ID.  The details in the appendix will indicate what you should provide in this field.

-	Most command types require a command ID number.  This indicates the specific command to be performed (such as which X-10 command to perform or which output port command to perform).  Most of the available commands are identical to the commands you could include in your schedule.  In other words, the same commands you see listed in the actions entry screen for each command type are available here.  Refer to the appendix for the ID numbers and details of each command.  Refer to the userÕs manual for descriptions of what each command does.

¥	ÒRESPONSE ID #Ó

Each command has a unique ID Number (we interchangably refer to this as the Command ID # or the Response ID #).  This value is transmitted by the controller in response to the command.  It is a two-digit hex number.  You can read this value to verify that the proper command was performed by the controller.

¥	ÒRESPONSE DATAÓ

This column describes the data that will be transmitted by the controller in response to the command.  Only a few commands (primarily those to read the current state of an object) provide response data.  However, the controller always provides a acknlwledgement of all commands, as described in ÒCommand ProcessingÓ.

COMMAND DETAILS

Table 1 shows the format of each command.  However it doesnÕt give the details of exactly what the command data should be.  For commands that require it, this information is provided in the appendix.

EXAMPLES

The appendix contains examples for each command which is described there.

EVENT REPORTING MESSAGES FROM HOMEVISION-PC

When the controller is configured to report events, it will transmit a new message every time the event occurs.  The message follows the following format:

XX <Message> {CR} {LF} {Terminator}

¥	ÒXXÓ is the Message ID Number.  Each type of report message from the controller has a unique ID number in a 2-digit hexadecimal format.  Each digit will be a hexadecimal value (numbers 0-9 or letters A-F).  When converted to decimal format, they represent the numbers 0 through 255.  By reading these two bytes, a user program can quickly identify the type of message from the controller.

¥	Ò<Message>Ó will contain a text description of the event.

¥	{CR} signifies that a carriage return is sent.

¥	{LF} signifies that a line feed is sent.

¥	{Terminator} signifies that a single-byte terminator code is sent.  The byte has a value 1d, which is the ASCII code SOH (this is NOT the number 1, which is ASCII value 49d).  The terminator is provided to make it easier for a computer program to read and interpret the messages from the controller.

NOTE:  The carriage return and line feed do not actually appear on the screen if youÕre using a terminal emulation program.  Rather, they cause the cursor to move to the beginning of the next line.  The terminator code (ASCII code SOH) also will not appear on some terminal programs (although other programs may display it as a special symbol).

Table 2 shows the format of each message and gives examples.

TABLE 1

SEE FILE "TABLE1"

TABLE 2

SEE FILE "TABLE2"

APPENDIX

SEE FILE "APPENDIX"